Real time embedded systems

# Real time embedded systems

Lecture

Hardware and software timer

Timers

Hardware timers

Capture mode for timers

PWM

Pros and cons

Software timer

Lab

Topic

Coursework

Timers

- By alex

- Power point https://ports-main-mdl-euwest2.s3.eu-west-2.amazonaws.com/f6/d7/f6d7402145d6702630bf8eeb8d365c4df83cb823?response-content-disposition=inline%3B%20filename%3D%22Timer_verified.pdf%22&response-content-type=application%2Fpdf&X-Amz-Content-Sha256=UNSIGNED-PAYLOAD&X-Amz-Algorithm=AWS4-HMAC-SHA256&X-Amz-Credential=AKIAWRN6GJFLZ774IX4M%2F20260319%2Feu-west-2%2Fs3%2Faws4_request&X-Amz-Date=20260319T103743Z&X-Amz-SignedHeaders=host&X-Amz-Expires=21557&X-Amz-Signature=6ba4724d8ebc6297202cc83d519281594772d96bd14b6ede6e50797704e787bb

- Or see

- My github here() i will check the power points and take bits to work as assistance

# Lecture

## Hardware and software timer

### Timers

- Why

- Timers act as heart beats for the system enabling systems to accurately respond to time dependent systems

- Embedded

- Abs breaks wheel speed measurements  have a precision of microseconds.

- Smart thermo stats have an accuracy of seconds.

- These are examples we should understand that not all timers need same precision.

- Approach 1

- While loops / for loops these waste time and cpus are locked in

- Apporach 2

- Hardware timer + interupt this leaves the CPU to do what it wants while the function is interrupted.

- Most systems have a watchdog timer this timer resets the entier cpu/mcu if its ever locked or stuck this timer simply expects to be updated if its not then it will reset it does not care that you are doing something as such you add time to this timer regularly for operating systems this is less of an issue as it handles the watchdog timer.

#### Watchdog timers

- https://en.wikipedia.org/wiki/Mars_Pathfinder

- https://en.wikipedia.org/wiki/Priority_inversion

- this is a bug that was caused by a poorly designed watch dog timer

### Hardware timers

- Their are multiple types of hardware timer depending on process and processor

- For esp32 their are two types

- Watchdog

- General purpose

- You can use the general purpose but not the watchdog for processing generally.

- Hardware timers have 4 parts

- Clock source

- Prescaler

- Counter register

- Compare register

#### Comps

##### Clock source

- Internal oscilator with a defiend freq

- normaly an external crystal with a clock pin

##### Prescaler

- Divides the clock to a different rate normaly timer  freq = cloc_freq/ prescaler.

- For example we want 10khz 72MHz/7200 = 10khz so 0.1ms

###### Speed control

- 1 division = same speed as the clock so by pass

- if 72mhz

- 72 = 1mhz micorsecond timing

- 7200 = 10 khz  millisecond events

- 72000 1khz second range timing

- Define the fastest the system will ever need to respond and then decide the prescaler you need if its too fast your wasitng cpu time if your too slow you miss the deadline

##### Counter register

- Counter register counts up or down on each tick

- This could be 8 16 or 32 bits

###### Bit width matter

- 8 bit 256 bit count at prescaler of 1 it lasts 3.6 us 233ms

| - Counter width | - Max count | - Prescaler =1 | - Prescaler at 65536 |
| --- | --- | --- | --- |
| - 8 | - 256 | - 3.6us | - 233 ms |
| - 16 | - 65,536 | - 0.91ms | - 59.6 s |
| - 32 | - 4,294,967,296 | - 59.6 s | - ~3.9 million s |

- a large number of mcus use arm cortext m mcus have 3 - 12 timers depending on the cor you pick.

- TIM = Timer  (so tim1 is timer 1)

- STM32F103

- TIM1,TIM8: 16bit advanced timers

- TIM2,TIM3,TIM4: 16bit general purpose

- SysTick: 24bit system timer used by rtos

##### Compare register

- When the counter  == the compare register we reset the counter register and trigger the hardware timer

#### How it counts

- T period = (prescaler * (autoReload +1) )/fclock

- When the auto reload triggers it causes a pulse(i think bit confused by his explination)

#### Timer modes

- Need to figure out which a processor is other wise the system will not work properly

- Count up mode (most common)

- 0 ->1 ->2 and so in this one is  easiest to use and very common

- Count down mode

- Center alligned mode (used for advanced PWM)

- Counts to max compares if it matches the max and then counts down and monitors the pulse (think)

- This

#### Configuration example

- // configure TIM2 for stm32

- RCC->APV1ENR |= RCC_APB1ENR_TIM2EN

- TIM

### Capture mode for timers

- This is used for measuring things so

- Capture the rpm of a system

- At each rising edge you capture a bit of data and then  you can do understand the speed of the system(maybe again he lost me)

### PWM

#### Generating pwm waveforms

- Const uint8_t sien_table[64] ={128,140,153};

- Volatile uint8_t sample_index =0;

#### Pwm applications

- Fast motors

- Typical freqs between 10khz and 25khz

### Pros and cons

- use hardware timers where you need precision use software where you don't

#### Limits

- Their are a limtied number of timers on each mcu so use with acre

- Their complex to configure

- Can be hard to manage

- Each timer is more power and more pcb cost

#### benefits

- Precice timig with high resolution

- Operates independently of cpu exectuion

### Software timer

- Onr hardware timer can drive all software timers

- Each software timer is just a data structure in memory

- The rtos  kernel checks expiry on each tick

- Virtually unlimited in number(memory limited)

#### How they operate

- Tick interrupt fires every 1ms

- Timere services wakes up

- Timer 1 target tick 1500 , counter 1500 pulse high

#### Types

##### Oneshot

- Used onces and then it is dormant

##### Auto reload

- Cyclic does not go dormant it is like an alarm clock

#### States

##### dormant

- Does not check the call back again it is done and will not fire

##### running

- Timer is counting down and will send call backs when requested.

#### Creating a timer

##### Task

##### Example

- TimmerHandle_t xLedTimer;

- XledTimer = xTimerCreate(

- "LEDTIMER",

- pdMS_TO_TICKS(500,

- pdTrue,

- NULL,

- vLedtimercallback,

- );

##### Call back function

- Keep it short and fast  don't call delays

- They are non blocking fucntions if they block then the system will miss its spec

#### Timer service

- Also know as the daemon task or timer service daemon

##### Config for rtos

##### Timer queue

- Timer service commands

- start timer

- Stop timer

- Reset timer

- Change timer

# Lab

## Topic

# Coursework

## Timers

- Expect these to appear in the course work its something critical to the system